Power BI & PPT - Practical Case Study

Context:
This wind farm consists of 9 wind turbines. The SCADA system records average 10-minute values for several parameters for each turbine. The objective of this practical exercise is to use this data to build relevant KPIs and visualize the performance of the wind farm through Power BI (please organize and design the dashboard in a clean and aesthetic way, as if it was to be presented to a client.). Besides, create a 2 slides (PPT) analyzing the performance.

Power BI Dashboard

Prepare a Power BI dashboard using the table '10MIN' that includes the following elements:

- A filter to select one or several wind turbines.

- A power curve graph (Power vs Wind Speed) for the selected turbines, allowing comparison between them. If possible, as an additional improvement, allow the background display of two theoretical power curves for comparison purposes

- A time series graph showing:

  - The actual power produced by the selected turbines every 10 minutes.

  - The theoretical power (ideal curve) for the same timestamps.

- A KPI card calculating the Time-Based Availability, defined as: Total time the wind turbine has been operating correctly divided by the total time it should have been operating. Downtime is considered as the time when a turbine has zero production while wind conditions indicate it should have been producing.

- A KPI cards calculating:

  - The energy lost due to stoppages.

  - The Energy-Based Availability = Energy produced / (Energy produced + Energy lost due to stoppages).

- Prepare a summary table that aggregates the following KPIs, by turbine and by month:

  - Average Wind Speed [m/s]

  - Total Energy Produced [MWh]

  - Energy-Based Availability [%]

  - Time-Based Availability [%]

  - Scalability Requirement

- Scalability: Prepare the Power BI solution so that if new turbines are added in the dataset (for example, moving from 9 to 11 turbines), the data model and visualizations automatically incorporate the new turbines without the need for manual modification. This should be achieved through appropriate Power Query transformations and data modeling.

Performance Analysis  & Improvement Proposals (PPT)

After building the dashboard and the KPIs, provide a brief analysis of the wind farm performance:

- Are there turbines that clearly perform worse than others?

- Do the power curves of some turbines deviate significantly from the expected curve?

- Are there periods with relevant downtime that deserve further investigation?

Based on the data available:

- What other KPIs or analysis would you propose to improve the monitoring of the wind farm performance?

- Would you suggest adding new data sources (alarms, status codes, maintenance..)